Chapter 2

Digital and Analog I/O

Experiment 1 - Blinking LED	1

Experiment 2 - Turning On An LED	4

Experiment 3 - LED with Changing Blink Rate	4

Experiment 4 - Fading LED	5

Experiment 5 - Simple LED Control With a Button	8

Course Plan & Teachers	10

Link: https://www.electronicshub.org/getting-started-with-stm32f103c8t6-blue-pill/ Link: https://electropeak.com/learn/how-to-program-stm32-blue-pill-stm32f103c8t6-with-arduino-ide/ Link: https://stm32-base.org/assets/pdf/boards/original-schematic-STM32F103C8T6-Blue_Pill.pdf Datasheet: https://www.st.com/resource/en/datasheet/stm32f103c8.pdf Reference Manual: https://www.st.com/resource/en/reference_manual/rm0008-stm32f101xx-stm32f102xx-stm32f103xx-stm32f105xx-and-stm32f107xx-advanced-armbased-32bit-mcus-stmicroelectronics.pdf

# Experiment 1 - Blinking LED void setup() { // put your setup code here, to run once: pinMode(PC13, OUTPUT); } void loop() { // put your main code here, to run repeatedly: digitalWrite(PC13, HIGH); delay(1000); digitalWrite(PC13, LOW); delay(1000); }.

220Ω resistor

List of some of the digital output pins commonly used on the Blue Pill: - PA0 - PA15 (Port A, Pins 0-15) - PB0 - PB15 (Port B, Pins 0-15) - PC13 - PC15 (Port C, Pins 13-15) List of digital output pins available on the Arduino Uno: - Digital Pins 0 to 13: These pins can be used as digital input or output pins. Digital Pins 0 and 1 are also connected to the hardware serial interface (RX and TX), respectively. - Digital Pins A0 to A5: These pins can be used as digital input or output pins, in addition to their analog input functionality.

# Experiment 2 - Turning On An LED void setup() { pinMode (PA15, OUTPUT); // Set the LED pin as an output digitalWrite(PA15, HIGH); // Set the LED pin high } void loop() { // We are not doing anything in the loop! }.

# Experiment 3 - LED with Changing Blink Rate void setup() { pinMode (PA15, OUTPUT); // Set the LED pin as an output } void loop() { for (int i=100; i<=1000; i=i+100) { digitalWrite(PA15, HIGH); delay(i); digitalWrite(PA15, LOW); delay(i); } }.

Pulse-Width Modulation with analogWrite()

List of analog output (PWM) pins commonly used on the Blue Pill: - PA0 (Timer2 Channel 1) - PA1 (Timer2 Channel 2) - PA2 (Timer2 Channel 3) - PA3 (Timer2 Channel 4) - PA6 (Timer3 Channel 1) - PA7 (Timer3 Channel 2) - PB0 (Timer3 Channel 3) - PB1 (Timer3 Channel 4) - PB6 (Timer4 Channel 1) - PB7 (Timer4 Channel 2) List of digital output pins available on the Arduino Uno: - Digital Pins 3, 5, 6, 9, 10, and 11: These pins support PWM functionality and can be used for analog output via PWM.

# Experiment 4 - Fading LED void setup() { pinMode (PB0, OUTPUT); //Set the LED pin as an output } void loop() { for (int i=0; i<256; i++) { analogWrite(PB0, i); delay(10); } for (int i=255; i>=0; i--) { analogWrite(PB0, i); delay(10); } }.

Reading Digital Inputs

List of digital input pins commonly used on the Blue Pill: - PA0 - PA15 (Port A, Pins 0-15) - PB0 - PB15 (Port B, Pins 0-15) - PC0 - PC15 (Port C, Pins 0-15) List of digital input pins available on the Arduino Uno: - Digital Pins 0 to 13: These pins can be used as digital input or output pins. Digital Pins 0 and 1 are also connected to the hardware serial interface (RX and TX), respectively. - Digital Pins A0 to A5: These pins can be used as digital input or output pins, in addition to their analog input functionality.

# Experiment 5 - Simple LED Control With a Button const int LED=PB0; // The LED is connected to pin 9 const int BUTTON=PC15; // The Button is connected to pin 2 void setup() { pinMode (LED, OUTPUT); // Set the LED pin as an output pinMode (BUTTON, INPUT); // Set button as input (not required) } void loop() { if (digitalRead(BUTTON) == LOW) { digitalWrite(LED, LOW); } else { digitalWrite(LED, HIGH); } }.

# Experiment 6 - Reading temperature sensor

// Temperature Alert!

const int BLED=PB0; // Blue LED Cathode on Pin B0

const int GLED=PB1; // Green LED Cathode on Pin B1

const int RLED=PB2; // Red LED Cathode on Pin B2

const int TEMP=PA0; // Temp Sensor is on pin A0

const int LOWER_BOUND=139; // Lower Threshold

const int UPPER_BOUND=147; // Upper Threshold

int val = 0; // Variable to hold analog reading

void setup() {

pinMode (TEMP, INPUT);

pinMode (BLED, OUTPUT); // Set Blue LED as Output

pinMode (GLED, OUTPUT); // Set Green LED as Output

pinMode (RLED, OUTPUT); // Set Red LED as Output

}

void loop() {

val = analogRead(TEMP);

if (val < LOWER_BOUND) { // LED is Blue

digitalWrite(RLED, HIGH);

digitalWrite(GLED, HIGH);

digitalWrite(BLED, LOW);

}

else if (val > UPPER_BOUND) { // LED is Red

digitalWrite(RLED, LOW);

digitalWrite(GLED, HIGH);

digitalWrite(BLED, HIGH);

}

else { // LED is Green

digitalWrite(RLED, HIGH);

digitalWrite(GLED, LOW);

digitalWrite(BLED, HIGH);

}

}

# Experiment 7 - Reading photoresistor

// Automatic Night Light

const int WLED=PA0; // White LED Anode on pin PA0 (PWM)

const int LIGHT=PA1; // Light Sensor on Analog Pin PA1

const int MIN_LIGHT=200; // Minimum Expected light value

const int MAX_LIGHT=900; // Maximum Expected Light value

int val = 0; // Variable to hold the analog reading

void setup() {

pinMode(WLED, OUTPUT); // Set White LED pin as output

}

void loop() {

val = analogRead(LIGHT); // Read the light sensor

val = map(val, MIN_LIGHT, MAX_LIGHT, 255, 0); // Map the light reading

val = constrain(val, 0, 255); // Constrain light value

analogWrite(WLED, val); // Control the White LED

}

# Course Plan & Teachers

Nahin Ul Sadad, Assistant Professor, CSE, RUET

Md. Farhan Shakib, Lecturer, CSE, RUET

Md. Nasif Osman Khansur, Lecturer, CSE, RUET