Led blinking:

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV AL,0000H

MOV DX,PORTA

OUT DX,AL

MOV CX,0DF36H

XXX:

LOOP XXX

LOOPY1:

MOV AL,00FFH

MOV DX,PORTA

OUT DX,AL

MOV CX,0DF36H

LOOPY2:LOOP LOOPY2

JMP XX

CODE ENDS

END

1 Push button on-off:

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTB

IN AL,DX

CMP AL,1

JE ON

MOV AL,0

MOV DX,PORTA

OUT DX,AL

JMP XX

ON:

MOV AL,1

MOV DX,PORTA

OUT DX,AL

JMP XX

CODE ENDS

END

2 push button on-off:

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTC

IN AL,DX

MOV BL,AL

MOV DX,PORTC

IN AL,DX

MOV CL,AL

CMP BL,10000000B

JE ON

CMP CL,01000000B

JE OFF

JMP XX

ON:

MOV AL,0FFH

MOV DX,PORTA

OUT DX,AL

JMP XX

OFF:

MOV AL,0

MOV DX,PORTA

OUT DX,AL

JMP XX

CODE ENDS

END

2 push button(xor operation):

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTC

IN AL,DX

MOV BL,AL

MOV DX,PORTC

IN AL,DX

MOV CL,AL

CMP BL,10000000B

JE ON

CMP CL,01000000B

JE OFF

JMP XX

ON:

MOV AL,0FFH

MOV DX,PORTA

OUT DX,AL

JMP XX

OFF:

MOV AL,0

MOV DX,PORTA

OUT DX,AL

JMP XX

CODE ENDS

END

2 push button (blinking rate high-low):

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

MOV BX,0DF36H

JMP XX

XX:

MOV DX,PORTB

MOV AL,0FFH

OUT DX,AL

MOV CX,BX

RUN:

LOOP RUN

MOV DX,PORTB

MOV AL,0H

OUT DX,AL

MOV CX,BX

RUN2:

LOOP RUN2

MOV DX,PORTA

IN AL,DX

MOV CL,AL

MOV DX,PORTA

IN AL,DX

MOV CH,AL

CMP CL,00000010B

JE HH

CMP CH,00000100B

JE LL

JMP XX

HH:

ADD BX,10000

JMP XX

LL:

SUB BX,10000

JMP XX

CODE ENDS

END

1 push button(debounce):

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

MOV BL,0B

JMP XX

XX:

MOV DX,PORTB

MOV AL,BL

OUT DX,AL

MOV DX,PORTA

IN AL,DX

CMP AL,00000010B

JE RECHECK

JMP XX

RECHECK:

MOV CX,2200

CHECK:

LOOP CHECK

MOV DX,PORTA

IN AL,DX

CMP AL,00000010B

JE DEBOUNCE

JMP XX

DEBOUNCE:

XOR BL,00000010B

JMP XX

CODE ENDS

END

2 push button (XNOR):

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTC

IN AL,DX

MOV BL,AL

MOV DX,PORTC

IN AL,DX

MOV CL,AL

CMP BL,11000000B

JE ON

CMP BL,0B

JE ON

CMP BL,10000000B

JE OFF

CMP CL,01000000B

JE OFF

JMP XX

ON:

MOV AL,0FFH

MOV DX,PORTA

OUT DX,AL

JMP XX

OFF:

MOV AL,0

MOV DX,PORTA

OUT DX,AL

JMP XX

CODE ENDS

END

2 push button-2 LED:

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTC

IN AL,DX

MOV BL,AL

MOV DX,PORTC

IN AL,DX

MOV CL,AL

CMP BL,01000000B

JE ON1

CMP BL,11000000B

JE ON1

JNE OFF1

A1:

CMP BL,10000000B

JE ON2

CMP BL,11000000B

JE ON2

JNE OFF2

ON1:

MOV AL,00001000B

MOV DX,PORTA

OUT DX,AL

JMP A1

ON2:

MOV AL,00100000B

MOV DX,PORTB

OUT DX,AL

JMP XX

OFF1:

MOV AL,0B

MOV DX,PORTA

OUT DX,AL

JMP A1

OFF2:

MOV AL,0B

MOV DX,PORTB

OUT DX,AL

JMP XX

CODE ENDS

END

1 PUSH BUTTON 2 LED:

DATA SEGMENT

PORTA EQU 00H

PORTB EQU 02H

PORTC EQU 04H

PORT_CON EQU 06H

DATA ENDS

CODE SEGMENT

MOV AX,DATA

MOV DS,AX

ORG 0000H

START:

MOV DX, PORT_CON

MOV AL,10000000B

OUT DX,AL

JMP XX

XX:

MOV DX,PORTB

IN AL,DX

CMP AL,1B

JE ON

MOV AL,0B

MOV DX,PORTA

OUT DX,AL

MOV AL,00010000B

MOV DX,PORTC

OUT DX,AL

JMP XX

ON:

MOV AL,1B

MOV DX,PORTA

OUT DX,AL

MOV AL,0B

MOV DX,PORTC

OUT DX,AL

JMP XX

CODE ENDS

END

Timer(OPM):

void TimerOPM_init()

{

RCC->APB1ENR |= 1UL<<0;

TIM2->CNT = 0;

// Prescaler value

TIM2->PSC = 3600 - 1;

// Auto-reload value for 1000 ms

TIM2->ARR = 10000 - 1;

// Configure as OPM

TIM2->CR1 |= 1UL<<3;

}

/*while(1) {

// Timer OPM er while loop.....

TIM2->CR1 |= 1; // Start Timer

while(TIM2->CR1 & 1); // Check if still counting

GPIOA->ODR ^= 1UL<<0;  // Toggle output

}*/

Timer(OCM):

void TimerOCM_init()

{

RCC->APB1ENR |= (1UL << 0);  // TIM2 clock enable

// Configure TIM2

// Prescaler value (PSC)

TIM2->PSC = 3600 - 1;

// Auto-reload value (ARR) for 1 second timing (assuming 36MHz clock)

TIM2->ARR = 10000 - 1;

// Set the capture/compare register value for CH1 (50% duty cycle)

TIM2->CCR1 = 5000 - 1;

// Set Output Compare mode to Toggle (011)

TIM2->CCMR1 |= 3<<4;

// Enable the output on CH1

TIM2->CCER |= 1<<0;

TIM2->CR1 |= 1;

}

/*while(1)

{

// Timer OCM er while loop........

}*/