- Problem: Signal Convolution using Unit Impulse Signal

Logical Solution:

- x(n)=…….a1δ(n+2)+a2δ(n+1)+a3δ(n)+a4δ(n-1)+a5δ(n-2)+………….

- The output of LTI system of impulse signal is h(n).

- y(n)= …….a1h(n+2)+a2h(n+1)+a3h(n)+a4h(n-1)+a5h(n-2)+………….

Source Code:

import numpy as np import matplotlib.pyplot as plt x_n=np.array([2,0,2]) x_t=0 h_n=[1,1,1] h_t=1 length=len(x_n)+len(h_n)-1 for i in range(length-len(h_n)+1): h_n.append(0) h_n=np.array(h_n) print(length,h_n) Output-h(n) # i) 2del(n) t=np.array([ i-x_t for i in range(len(x_n))]) del_n=np.array([0 for i in range(len(x_n))]) del_t=x_t for i in t: if i==0: del_n[i+x_t]=x_n[i] else: del_n[i]=0 del_n=np.array(del_n) plt.grid(True) plt.stem(t,del_n) plt.show() Figure-2δ(n) # ii) 2h(n) h1=h_n*del_n[0] t=np.array([ i-h_t for i in range(len(h_n))]) plt.grid(True) plt.stem(t,h1) plt.show() Figure-2h(n) # iii) 2del(n-2) t=np.array([ i-x_t for i in range(len(x_n))]) del_n=np.array([0 for i in range(len(x_n))]) print(del_n) del_t=x_t for i in t: if i==2: del_n[i+x_t]=x_n[i] else: del_n[i]=0 print(del_n) del_n=np.array(del_n) print(del_n,t) plt.grid(True) plt.stem(t,del_n) plt.show() Figure-2δ(n-2) # iv) 2h(n-2) for i in range(2): h_n=h_n[:-1] h_n=np.insert(h_n,0,0) h2=h_n*del_n[2] print(h2) t=np.array([ i-h_t for i in range(len(h_n))]) plt.grid(True) plt.grid(True) plt.stem(t,h2) plt.show() Figure-2h(n-2) # v) h(n)+h(n-2) y_n=h1+h2 print(y_n) t=np.array([ i-h_t for i in range(len(h_n))]) plt.grid(True) plt.stem(t,y_n) plt.show() Figure-2h(n)+2h(n-2)

Discussion:

- Firstly, determine each unit impulse signal from the original signal

- Then send the impulse signal to the LTI system to get h(n), response signal

- Problem: Signal Convolution using Summation Formula

Logical Solution:

- y(n)=k=-∞∞xn*h(n-k)

Source Code:

import numpy as np import matplotlib.pyplot as plt x_n=np.array([1,2,3]) # x_n=np.array([1,2,4,8]) lenx=len(x_n) x_t=0 h_n=np.array([4,5,6]) h_t=1 y_t=h_t length=len(x_n)+len(h_n)-1 h_n=h_n[::-1] for i in range(len(h_n)): x_n=np.insert(x_n,0,0) for i in range(len(h_n)): x_n=np.append(x_n,0) l=length-len(h_n)-h_t+1 for i in range(l): h_n=np.append(h_n,0) h_t-=1 for i in range(len(x_n)-len(h_n)): h_n=np.append(h_n,0) print(h_n,x_n,h_t) Output- h(n), x(n), h(t) mn=min(h_t,x_t) t=[] y_n=[] for i in range(mn,length+2): y_n.append(0) t.append(i-1) for j in range(len(h_n)): y_n[-1]+=x_n[j]*h_n[j] h_n=h_n[:-1] h_n=np.insert(h_n,0,0) h_t+=1 plt.stem(t,y_n) plt.grid(True) plt.show() Output- Convoluted Signal

Discussion:

- Firstly, I have reversed the response signal.

- Secondly, left shift that signal for proper calculation.

- Then shift that signal and multiply the values and sum it.

- Problem: Convolution Using Built-in Function (0/1 Signal)

Logical Solution:

- y(n)=k=-∞∞xn*h(n-k)

Source Code:

import numpy as np import matplotlib.pyplot as plt x_n=[1,1,1,1,1,0,0,0,0,0,1,1,1,1,1,0,0,0,0,0] h_n=[.2,.2,.2,.2,.2] plt.stem(x_n) plt.show() Fig- 0/1 Signal y_n=np.convolve(x_n,h_n) plt.stem(y_n) plt.show() Output- Convoluted Signal

Discussion:

- Here I have used built-in numpy function which is numpy.convolve(x,h).

- Problem: Convolution Using Built-in Function (Sine Signal)

Logical Solution:

- y(n)=k=-∞∞xn*h(n-k)

Source Code:

import numpy as np import matplotlib.pyplot as plt n=np.linspace(-1,1,1000) x_n=np.sin(2*np.pi*n) plt.plot(x_n) plt.show() Fig- Sine Signal noise=np.random.rand(len(n)) noisy=x_n+noise plt.plot(noisy) plt.show() Output- Noisy Sine Signal filtered=np.ones(100)/100 after_filter=np.convolve(noisy,filtered) plt.plot(after_filter) plt.show() Fig- Filtered Signal

Discussion:

- After adding noise to original sinusoidal signal, pass it with the filtered value to the numpy built in function for convolution.