FPGA-Based Hardware Acceleration of a Low-Power Systolic Array Architecture Design for CNN Image Classification

Here is the step-by-step working procedure for a CNN thesis on an FPGA, broken down into four distinct phases.

Phase 1: Software Development & Optimization (The AI Phase)

This is entirely done on your PC using standard ML libraries.

- Select the Dataset and Model: Choose a manageable dataset (like CIFAR-10 for images or MIT-BIH for ECG signals). Design a lightweight CNN architecture (e.g., 2-3 convolutional layers, 1 max-pooling layer, 1 dense layer).

- Train in FP32: Train the baseline model in Python using PyTorch or TensorFlow. This uses 32-bit floating-point numbers. Achieve your target accuracy (e.g., 90%).

- Quantization-Aware Training (QAT): This is the most critical software step. FPGAs hate floating-point math. You must use tools like Brevitas (for PyTorch) or QKeras (for TensorFlow) to retrain your network so the weights and activations are constrained to 8-bit integers (INT8) or even lower (like 4-bit or 1-bit).

- Export the Model: Once the quantized model is trained and verified, export it.

Phase 2: Hardware Translation (The VLSI Phase)

This is where software becomes digital logic.

- High-Level Synthesis (HLS): Writing raw Verilog for a whole CNN is generally too complex for a standard timeline. Instead, use an open-source tool called hls4ml. You feed your quantized Python model into hls4ml, and it automatically generates C++ code optimized for hardware.

- C-Simulation & Synthesis: Use a tool like Xilinx Vitis HLS.

  - First, run a C-Simulation to ensure the C++ code produces the exact same predictions as your Python model.

  - Then, run Synthesis. The tool translates the C++ into Register-Transfer Level (RTL) code (Verilog or VHDL).

- RTL Verification: Standard RTL verification practices are essential here. You will generate a testbench to simulate the Verilog waveforms and prove that the digital logic functions correctly before touching physical hardware.

Phase 3: Hardware Implementation (The Engineering Phase)

This maps your logic gates to the physical silicon.

- IP Block Generation: Package your synthesized Verilog CNN into an Intellectual Property (IP) block.

- System Integration (Vivado): Open Xilinx Vivado (or Intel Quartus). Create a block design. You will connect your custom CNN IP block to the FPGA's Processing System (the ARM core) using an AXI Stream Interface (a high-speed data bus).

- Synthesis and Place & Route (P&R): Instruct the software to map your logic to the specific physical slices of your FPGA board. This step can take hours to compile.

- Bitstream Generation: The output of P&R is a .bit file. This is the binary configuration file that physically wires the FPGA's logic gates.

Phase 4: Board Deployment & Testing (The Edge Phase)

Running real data through the physical chip.

- Deploy to the Board: If using a board like the PYNQ-Z2, you boot it up, open a Jupyter Notebook directly on the board, and use Python to load your .bit file into the FPGA fabric.

- Run Inference: Send a test image from the ARM processor over the AXI bus into your custom FPGA hardware, and read the prediction that comes back.

- Measure and Record (The Thesis Data): This is what you actually write your thesis about. You must extract three key metrics:

  - Performance (Latency): Measure the exact microseconds it takes to process one image.

  - Area (Resource Utilization): Record how much of the FPGA you consumed (e.g., "Used 85% of DSP slices, 60% of BRAM").

  - Power: Measure the active power consumption in watts.

The Typical Timeline

For a student who already has foundational knowledge in Python and digital logic, the timeline usually looks like this:

- Month 1: Phase 1 (Training & Quantizing the model).

- Month 2: Phase 2 & 3 (HLS, Vivado integration, fighting software errors).

- Month 3: Phase 4 (Deploying to the board, fixing AXI bus data-formatting bugs, and recording the final metrics).

To get started on Phase 1, do you have a preference between using PyTorch or TensorFlow/Keras for building the initial software model?

Image classification is not a "systolic array problem." Rather, a systolic array is the hardware solution to the heavy math required by image classification.

Here is how the three layers connect:

- The Problem (The Task): Image Classification. You want the computer to look at a 32x32 grid of pixels (like CIFAR-10) and identify a "ship."

- The Algorithm (The Math): Convolutional Neural Networks (CNNs). To find the ship, a CNN slides mathematical filters over the image. This requires millions of matrix multiplications (multiplying rows of numbers by columns of numbers).

- The Hardware (The Architecture): The Systolic Array. You could do those matrix multiplications on a standard CPU, one by one. But a systolic array is a physical grid of processors built specifically to compute massive matrix multiplications in parallel.

A Helpful Analogy

Think of a real-world construction project:

- The Problem: You need to dig a massive foundation for a skyscraper (Image Classification).

- The Algorithm: The method of moving dirt (CNN).

- The Hardware: A Systolic Array is like a massive industrial excavator. A standard CPU is like a single worker with a shovel.

You don't have to use an excavator to dig a hole—a shovel works, it's just incredibly slow. Similarly, you don't have to use a systolic array for image classification, but because image classification requires so much heavy math, a systolic array is the most efficient physical architecture to get the job done quickly and with low power.